T.Y .BTech (Electronics) – Image Processing

Expt No -5: Image Sharpening

Problem Statement –

    - Write a program for image sharpening using low pass filters and high pass filter in frequency domain.

- Objective: To analyze the effect of image Sharpening on image.

- Tools Required: MATLAB

- Theory: Brief describe in short

- Specify the syntax used in MATLAB for following sharpening operation.

.                   Hint : you tube link shared on classroom

- Conclusion:

- Discussion –

- Program 1: Write a program for image sharpening using low pass filters and high pass filter in frequency domain.

image = imread('butterfly.jpg');

if size(image, 3) == 3

image = rgb2gray(image);

end

F = fft2(double(image));

F_shifted = fftshift(F);

[M, N] = size(image);

[U, V] = meshgrid(1:N, 1:M);

D0 = 30;

D = sqrt((U - N/2).^2 + (V - M/2).^2);

H_low = exp(-(D.^2) / (2*(D0^2)));

H_high = 1 - H_low;

F_low = F_shifted .* H_low;

image_low_filtered = abs(ifft2(ifftshift(F_low)));

F_high = F_shifted .* H_high;

image_high_filtered = abs(ifft2(ifftshift(F_high)));

image_sharpened = double(image) - image_low_filtered;

subplot(2, 3, 1);

imshow(image, []);

title('Original Image');

subplot(2, 3, 2);

imshow(image_low_filtered, []);

title('Low-pass Filtered Image');

subplot(2, 3, 3);

imshow(image_high_filtered, []);

title('High-pass Filtered Image');

subplot(2, 3, 4);

imshow(image_sharpened, []);

title('Sharpened Image');

Output: